F4321_HS Dynamic Library Description Document

Time: 2025.2.25

  - Composition of dynamic libraries and supporting devices

| Dynamic library name | Dynamic library version | Dynamic library header file | Dynamic Library DLL Files | Dynamic library supports devices |
| --- | --- | --- | --- | --- |
| FL_DLP_HS_F4321_API | 1.0 | f4321_hs.h | F4321_HS.dll f4321_hs.o libF4321_HS.a | F4321_HSseries all equipment |

  - Feedback code table for dynamic library functions

In the dynamic library header file - f4321_hs. h, there is a dynamic library function feedback code table-FL_DLP_HS_S4321_RES_Type.

| Feedback Name | Feedback Value | Feedback meaning and partial solutions |
| --- | --- | --- |
| Res_Success | 0 | Feedback meaning：Normal feedback |
| Res_SocketWSAStartupFail | -100 | Feedback meaning：UDP WSAStartup Failed |
| Res_SocketCreateFail | -99 | Feedback meaning：UDP socket creation failed |
| Res_SocketNoBlockFail | -98 | Feedback meaning：UDP failed to set socket non blocking mode |
| Res_SocketBindFail | -97 | Feedback meaning：UDP bind Failed Solution： - Check if the device is powered on and also check if the Ethernet in the network adapter is connected properly - Check if the Ethernet IP of IPV4 in the network adapter is set correctly - Check if the port is occupied, enter netstat - ano ¦ findstr "xxxx" in the cmd command window, and replace xxxx with the actual port used |
| Res_UDPThreadCloseFail | -96 | Feedback meaning：UDP thread not properly closed |
| Res_RecThreadCloseFail | -95 | Feedback meaning：Rec thread not closed correctly |
| Res_CMDNoParam_CMDTypeError | -94 | Feedback meaning：CMD No Parameter error in type Solution：CMD No Parameter Command IDs are 1-8 |
| Res_CMDNoParam_CMDPackError | -93 | Feedback meaning：CMD No Parameter Assembly error |
| Res_CMDPlay1_CMDTypeError | -92 | Feedback meaning：CMD Play Command 1 error in type Solution：CMD Play Command 1 Command IDs are 10-17 |
| Res_CMDPlay1_ParamSPError | -91 | Feedback meaning：CMD Play Command 1 Image playback starting position parameter error Solution：This parameter starts from 1 and is set to n, representing playback starting from the nth binary image |
| Res_CMDPlay1_ParamPicNumError | -90 | Feedback meaning：CMD Play Command 1 The parameter for the number of images played is incorrect Solution：This parameter starts from 1 and is set to n, representing the playback of n binary images |
| Res_CMDPlay1_CMDPackError | -89 | Feedback meaning：CMD Play Command 1 Assembly error |
| Res_CMDSetKBXL_CMDTypeError | -88 | Feedback meaning：CMD Set Variable Sequence Command error in type Solution：CMD Set Variable Sequence Command 命令ID为9 |
| Res_CMDSetKBXL_SequencenumError | -87 | Feedback meaning：CMD Set Variable Sequence Command Sequence length parameter error Solution：CMD Set Variable Sequence Command The sequence length parameter should be between 1-8192 |
| Res_CMDSetKBXL_TimetypeError | -86 | Feedback meaning：CMD Set Variable Sequence Command Time type parameter error Solution：CMD Set Variable Sequence Command The time type parameter should be 0 or 1 |
| Res_CMDSetKBXL_CMDPackError | -85 | Feedback meaning：CMD Set Variable Sequence Command Assembly error |
| Res_CMDSetKBXL_TimeShowError | -84 | Feedback meaning：CMD Set Variable Sequence Command Exposure time parameter error Solution：CMD Set Variable Sequence Command The exposure time parameter is set too small. The minimum exposure time parameter can be viewed in the log file |
| Res_CMDHDMIParam_CMDTypeError | -83 | Feedback meaning：CMD Set HDMI Parameters Command error in type Solution：CMD Set HDMI Parameters Command Command ID is 18 |
| Res_CMDHDMIParam_TimeShowError | -82 | Feedback meaning：CMD Set HDMI Parameters Command Single image display time parameter error Solution：This parameter should be less than or equal to 3000000 (5ns) |
| Res_CMDHDMIParam_CMDPackError | -81 | Feedback meaning：CMD Set HDMI Parameters Command Assembly error |
| Res_CMDSSParam_CMDTypeError | -80 | Feedback meaning：CMD Set Gigabit/10G Ethernet Real time Play Parameter Command error in type Solution：CMD Set Gigabit/10G Ethernet Real time Play Parameter Command command ID is 19 |
| Res_CMDSSParam_GrayModeError | -79 | Feedback meaning：CMD Set Gigabit/10G Ethernet Real time Play Parameter Command Gray mode parameter error Solution：This parameter should be 0 or 1 |
| Res_CMDSSParam_TimetypeError | -78 | Feedback meaning：CMD Set Gigabit/10G Ethernet Real time Play Parameter Command Time type parameter error Solution：This parameter should be 0 or 1 |
| Res_CMDSSParam_TimeShowError | -77 | Feedback meaning：CMD Set Gigabit/10G Ethernet Real time Play Parameter Command Time parameter error |
| Res_CMDSSParam_CMDPackError | -76 | Feedback meaning：CMD Set Gigabit/10G Ethernet Real time Play Parameter Command Assembly error |
| Res_CMDSCCFParam_CMDTypeError | -75 | Feedback meaning：CMD Set Output Trigger Pulse Width Command error in type Solution：CMD Set Output Trigger Pulse Width Command Command ID is 20 |
| Res_CMDSCCFParam_CMDPackError | -74 | Feedback meaning：CMD Set Output Trigger Pulse Width Command Assembly error |
| Res_CMDSCCFParam_CFKDError | -73 | Feedback meaning：CMD Set Output Trigger Pulse Width Command Output trigger pulse width parameter error Solution：This parameter should be greater than or equal to 2000 (5ns) |
| Res_CMDSCCFParam_CFSRBYError | -72 | Feedback meaning：CMD Set Output Trigger Pulse Width Command Input trigger edge parameter error Solution：This parameter should be 0 or 1 |
| Res_CMDSCCFParam_CFSCBYError | -71 | Feedback meaning：CMD Set Output Trigger Pulse Width Command Output trigger edge parameter error Solution：This parameter should be 0 or 1 |
| Res_CMDSCCFParam_CFJGError | -70 | Feedback meaning：CMD Set Output Trigger Pulse Width Command Output trigger interval parameter error Solution：This parameter range is 1-65535 |
| Res_CMDSCCFParam_CFYSError | -69 | Feedback meaning：CMD Set Output Trigger Pulse Width Command Output trigger delay parameter error Solution：This parameter range is 0-16777215 (5ns) |
| Res_CMDPicShowParam_CMDTypeError | -68 | Feedback meaning：CMD Set Output Image Phase and Exposure Time Command error in type Solution：CMD Set Output Image Phase and Exposure Time Command Command ID is 21 |
| Res_CMDPicShowParam_CMDPackError | -67 | Feedback meaning：CMD Set Output Image Phase and Exposure Time Command Assembly error |
| Res_CMDPicShowParam_TimeShowError | -66 | Feedback meaning：CMD Set Output Image Phase and Exposure Time Command The exposure time is too short Solution：The exposure time is too short. You can check the minimum exposure time parameter in the log file |
| Res_CMDPicShowParam_TimeTypeError | -65 | Feedback meaning：CMD Set Output Image Phase and Exposure Time Command Time type parameter error Solution：This parameter should be 0 or 1 |
| Res_CMDPicShowParam_GrayTypeError | -64 | Feedback meaning：CMD Set Output Image Phase and Exposure Time Command Gray level parameter error Solution：This parameter range is 1-16 |
| Res_CMDDMDShowParam_CMDTypeError | -63 | Feedback meaning：CMD Set DMD Display Start Line and Display Line Count Command error in type Solution：CMD Set DMD Display Start Line and Display Line Count Command The command ID is 22 |
| Res_CMDDMDShowParam_CMDPackError | -62 | Feedback meaning：CMD Set DMD Display Start Line and Display Line Count Command Assembly error |
| Res_CMDDMDShowParam_NumSPError | -61 | Feedback meaning：CMD Set DMD Display Start Line and Display Line Count Command Display starting line parameter error Solution：This parameter range is from 1 to DMD with high resolution |
| Res_CMDDMDShowParam_NumShowError | -60 | Feedback meaning：CMD Set DMD Display Start Line and Display Line Count Command Display row number parameter error Solution：This parameter range is 1- (DMD resolution high - display starting line) |
| Res_CMDGDShowParam_CMDTypeError | -59 | Feedback meaning：CMD Set Image Scroll Display Parameter Command error in type Solution：CMD Set Image Scroll Display Parameter Command Command ID is 23 |
| Res_CMDGDShowParam_CMDPackError | -58 | Feedback meaning：CMD Set Image Scroll Display Parameter Command Assembly error |
| Res_CMDGDShowParam_GDTypeError | -57 | Feedback meaning：CMD Set Image Scroll Display Parameter Command Rolling type parameter error Solution：This parameter should be 0 or 1 |
| Res_CMDGDShowParam_GDSPError | -56 | Feedback meaning：CMD Set Image Scroll Display Parameter Command Rolling start position parameter error Solution：This parameter should be greater than or equal to 1 |
| Res_CMDGDShowParam_GDDCBJHError | -55 | Feedback meaning：CMD Set Image Scroll Display Parameter Command Rolling single step count parameter error Solution：This parameter should be greater than or equal to 1 |
| Res_CMDGDShowParam_GDBJCError | -54 | Feedback meaning：CMD Set Image Scroll Display Parameter Command The total number of rolling steps parameter is incorrect Solution：This parameter should be greater than or equal to 1 |
| Res_CMDPlayType_CMDTypeError | -53 | Feedback meaning：CMD setting image playback mode error in type Solution：CMD setting image playback mode Command ID is 24 |
| Res_CMDPlayType_CMDPackError | -52 | Feedback meaning：CMD setting image playback mode Assembly error |
| Res_CMDPlayType_PlayTypeError | -51 | Feedback meaning：CMD setting image playback mode Player type parameter error Solution：This parameter ranges from 0 to 4 |
| Res_CMDGDPlayParamm_CMDTypeError | -50 | Feedback meaning：CMD sets image scrolling playback parameters error in type Solution：CMD sets image scrolling playback parameters Command ID is 25 |
| Res_CMDGDPlayParamm_CMDPackError | -49 | Feedback meaning：CMD sets image scrolling playback parameters Assembly error |
| Res_CMDGDPlayParamm_GDTimeError | -48 | Feedback meaning：CMD sets image scrolling playback parameters Rolling single image display time parameter error Solution：The display time parameter is too small. You can check the minimum exposure time parameter in the log file |
| Res_JXThread_BMP_JXBMPDirError | -47 | Feedback meaning：Analyzing threads to parse BMP mode BMP storage folder address error Solution： - There is Chinese in the path - path does not exist |
| Res_JXThread_BMP_BMPOpenError | -46 | Feedback meaning：Analyzing threads to parse BMP mode BMP failed to open |
| Res_JXThread_BMP_BMPRead30Error | -45 | Feedback meaning：Analyzing threads to parse BMP mode BMP read 30 errors |
| Res_JXThread_BMP_BMPPicWError | -44 | Feedback meaning：Analyzing threads to parse BMP mode BMP width error Solution：Check if the width of the image matches the DMD resolution |
| Res_JXThread_BMP_BMPPicHError | -43 | Feedback meaning：Analyzing threads to parse BMP mode BMP height error Solution：Check if the height of the image matches the DMD display line count |
| Res_JXThread_BMP_BMPPicDError | -42 | Feedback meaning：Analyzing threads to parse BMP mode BMP bit depth error Solution：Check if the bit depth of the image matches the sending mode Binary transmission mode - bit depth is 1; 8-bit transmission mode - bit depth of 8 |
| Res_JXThread_BMP_BMPReadPYError | -41 | Feedback meaning：Analyzing threads to parse BMP mode BMP offset -30 error |
| Res_JXThread_BMP_BMPReadError | -40 | Feedback meaning：Analyzing threads to parse BMP mode BMP image data reading error |
| Res_JXThread_Bin_BinOpenError | -39 | Feedback meaning：Parse thread, parse Bin mode Bin opening failed |
| Res_JXThread_Bin_PicNumError | -38 | Feedback meaning：Parse thread, parse Bin mode Bin image number error Solution：Calculate whether the number of bytes in the Bin file matches the parameter of the number of images sent Is the Bin byte count equal to DMD resolution width * DMD display lines/8 * number of images sent |
| Res_JXThread_Bin_ReadPicError | -37 | Feedback meaning：Parse thread, parse Bin mode Bin reading image data error |

Tips:The above error does not have a solution. Please contact the after-sales engineer for clarification

  - Detailed description of dynamic library functions

Tips:

- The range of device IDs is 1, 2, and 3; The same dynamic library can control up to three devices simultaneously

- [in] represents input parameters; [out] represents output parameters

  - Function 1: Get Latest Operation Information Function - Correct Information

- The function in the header file displays：

- FL_DLP_HS_F4321_GetLastInfoRight(int DeviceID,unsigned char *Info_Right,int * Info_Length);

| - Function Function Description | - @brief:Get Latest Operation Information Function - Correct Information |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [out] Info_Right (Output) Latest operation information - correct information string. This parameter outputs English information and can be directly displayed - Example: C++， Apply for a 2048 byte char array and pass in an address pointer - @param [out] Info_Length (Output) Latest operation information - correct information string length, - Example: C++, Declare a variable of type int and pass in an address pointer |

  - Function 2: Get Latest Operation Information Function - Error Information

- The function in the header file displays：

- FL_DLP_HS_F4321_GetLastInfoError(int DeviceID,unsigned char *Info_Error,int * Info_Length);

| - Function Function Description | - @brief:Get Latest Operation Information Function - Error Information |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [out] Info_Error (Output) Latest operation information - Error message string. This parameter outputs English information and can be displayed directly - Example: C++， Apply for a 2048 byte char array and pass in an address pointer - @param [out] Info_Length (Output) Latest operation information - length of error message string, - Example: C++, Declare a variable of type int and pass in an address pointer |

  - Function 3: Initialize UDP Network Communication Function

- The function in the header file displays：

- FL_DLP_HS_F4321_Init(int DeviceID,char* server_addr,int server_port,char* target_addr,int target_port)

| - Function Function Description | - @brief:Initialize UDP network communication |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] server_addr (Input) Host computer IP, for example: 192.168.1.10 - @param [in] server_port (Input) Host computer port, example: 1234 - @param [in] target_addr (Input) Lower computer IP, for example: 192.168.1.100 - @param [in] target_port (Input) Lower computer port, example: 1234 |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

  - Function 4: Disable UDP Network Communication Function

- The function in the header file displays：

- FL_DLP_HS_F4321_DeInit(int DeviceID)

| - Function Function Description | - @brief:Close UDP network communication |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

  - Function 5: Send command function without parameters

- The function in the header file displays：

- FL_DLP_HS_F4321_CMDNoParam(int DeviceID,int CMDID)

| - Function Function Description | - @brief:Send command without parameters |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] CMDID (Input) Command ID - CMDID = 1 = CMD_Query, //Query Command - CMDID = 2 = CMD_Pause, //Play pause/continue command - CMDID = 3 = CMD_Stop, //Play termination command - CMDID = 4 = CMD_DMDFloat, //DMD Float command - CMDID = 5 = CMD_DMDReset, //DMD reset command - CMDID = 6 = CMD_DMDUPDown, //DMD image upside down - CMDID = 7 = CMD_DMDDatareverse, //DMD image data inversion； - CMDID = 8 = CMD_SFTrig, //Software simulation of external triggering |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

  - Function 6: Get Device Information Function

- The function in the header file displays：

- FL_DLP_HS_F4321_GetDeviceInfo(int DeviceID,unsigned char *DeviceInfo,int * DeviceInfolen)

| - Function Function Description | - @brief:Obtain device information - After sending the query command, call this function to obtain the device information |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [out] DeviceInfo (Output) Device information string, this parameter outputs English information, just output and display directly - Example: C++， Apply for a 2048 byte char array and pass in an address pointer - @param [out] DeviceInfolen (Output) Device information string length - Example: C++, Declare a variable of type int and pass in an address pointer |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

  - Function 7: Start playback function in normal cache mode/variable sequence mode

- The function in the header file displays：

- FL_DLP_HS_F4321_Play1(int DeviceID,int PlayType,int Play_SP,int Play_PicNum)

| - Function Function Description | - @brief Normal cache mode/Variable sequence mode start playback |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] PlayType (Input) Play type - PlayType = 10 = CMD_Play_InterDC //Internal single play - PlayType = 11 = CMD_Play_InterXH //Internal loop playback - PlayType = 12 = CMD_Play_ExterDC //External single play - PlayType = 13 = CMD_Play_ExterXH //External loop playback - PlayType = 14 = CMD_PlayXL_InterDC //Variable sequence internal single play - PlayType = 15 = CMD_PlayXL_InterXH //Variable sequence internal loop playback - PlayType = 16 = CMD_PlayXL_ExterDC //Variable sequence external single play - PlayType = 17 = CMD_PlayXL_ExterXH //Variable sequence external loop playback - @param [in] Play_SP (Input) Play starting position (unit: binary image), indicating that the device starts playing from the Xth binary image - Example: Assuming that the user has sent 100 binary images and the binary image storage starts from the first binary image, and wants to start playing from the 50th binary image, this parameter=50; - Assuming that the user has sent 100 8-bit images and the 8-bit images are stored starting from the first binary image, and wants to start playing from the 50th 8-bit image, this parameter=1+(50-1) * 8=393 - Tips:This parameter is not valid for variable sequences - @param [in] Play_PicNum (Input) The number of images played (unit: binary image), indicating that the device plays a total of X binary images - Example: Assuming the user wants to play 50 binary images, this parameter is 50; - Assuming the user wants to play 50 8-bit images, this parameter=50 * 8=400 - Tips:This parameter is not valid for variable sequences |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

- Tips：Due to the fact that the micro mirror of DMD essentially only has two states of switching, representing 0 and 1, which means binary images, binary images are used as counting units in the device. One 8-bit image is equal to eight binary images.

  - Function 8: Set Device Variable Sequence Function

- The function in the header file displays：

- FL_DLP_HS_F4321_SetKBXL(int DeviceID,int CMDID,int Sequence_num,int Time_type,unsigned int * Sequence_Add,unsigned int * Sequence_ShowTime,unsigned long long * Sequence_DarkTime)

| - Function Function Description | - @brief Set variable sequence |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] CMDID (Input) Command ID, - set device variable sequence command ID=9 - @param [in] Sequence_num (Input) Number of sequences，How many binary image variable sequences have been set by the user, with a parameter range of 1-8192 - Example: Assuming the user sets a variable sequence of 100 binary images, this parameter=100 - @param [in] Time_type (Input) Type of time calculation， - This parameter = 1 - @param [in] Sequence_Add (Input) Sequence image address, this parameter is an unsigned int array, the length of the array is Sequence_num, each unsigned int value is the storage location of the image in the lower computer device, the unit is a binary image, this parameter counts from 1 - Example: Assuming the variable sequence playback order set by the user is the second image, the fourth image, the first image, and the third image, - Sequence_Add[0] = 2;Sequence_Add[1] = 4; - Sequence_Add[2] = 1;Sequence_Add[3] = 3; - @param [in] Sequence_ShowTime (Input) Display time of sequence image, this parameter is an unsigned int array, the length of the array is Sequence_num , each unsigned int value is the display time of each variable sequence image, in units of 5ns - Example: Assuming the variable sequence display time set by the user is 1s, 2s, 3s, 4s; - Sequence_ShowTime[0] = 1000000000/5 = 200000000; - Sequence_ShowTime[1] = 2000000000/5 = 400000000; - Sequence_ShowTime[2] = 3000000000/5 = 600000000; - Sequence_ShowTime[3] = 4000000000/5 = 800000000; - @param [in] Sequence_DarkTime (Input) Sequence images: Dark time. This parameter is an unsigned long long array with a length equal to the number of sequences. Each unsigned long long value represents the dark time for each variable sequence image, with a unit of 5ns - Example: Assuming the variable sequence dark time set by the user is 1s, 2s, 3s, 4s; - Sequence_DarkTime[0] = 1000000000/5 = 200000000; - Sequence_DarkTime[1] = 2000000000/5 = 400000000; - Sequence_DarkTime[2] = 3000000000/5 = 600000000; - Sequence_DarkTime[3] = 4000000000/5 = 800000000; |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

- Tips:The variable sequence function is limited to binary graphs only

  - Function 9: Set HDMI real-time playback mode parameter function

- The function in the header file displays：

- FL_DLP_HS_F4321_SetParamHDMI(int DeviceID,int CMDID,unsigned int Time_Show)

| - Function Function Description | - @brief Set HDMI real-time playback parameter command |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] CMDID (Input) Command ID, Command ID=18 - @param [in] Time_Show HDMI (Input) Display time for each 8-bit image, range: 0-15ms, unit: 5ns, - Example: The user wants a display time of 10ms for each 8-bit image, and this parameter=10000000/5=2000000; |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

- Tips:HDMI real-time playback function is limited to 8-bit images only

  - Function 10: Set Gigabit/10G Ethernet real-time playback parameter function

- The function in the header file displays：

- FL_DLP_HS_F4321_SetParamSS(int DeviceID,int CMDID,int GrayMode,int Time_type,unsigned int Time_Show)

| - Function Function Description | - @brief Set real-time playback parameters for Gigabit/10G Ethernet |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] CMDID (Input) Command ID, Command ID=19 - @param [in] GrayMode (Input) Gray level mode: 0 = binary image; 1 = 8 bit grayscale image - Example: If the user wants to play binary images in real-time, this parameter = 0; The user wants to play 8-bit images in real-time, this parameter = 1; - @param [in] Time_type (Input) Type of time calculation，This parameter = 0 - @param [in] Time_Show (Input) Display time for each image (binary or 8-bit), unit: 5ns - Tips:This parameter function is temporarily unavailable, so setting the default value to 0 is sufficient |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

  - Function 11: Set Trigger Parameter Function

- The function in the header file displays：

- FL_DLP_HS_F4321_SetParamCF(int DeviceID,int CMDID,unsigned int CF_KD,int CF_SRBY,int CF_SCBY,int CF_JG,int CF_YS)

| - Function Function Description | - @brief Set trigger related parameters |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] CMDID (Input) Command ID,Command ID = 20 - @param [in] CF_KD (Input) Output trigger pulse width, unit: 5ns, value range:>= 10us - Example: Assuming the user wants to set the output trigger pulse width to 20us, this parameter=20000/5=4000; - Tips:This parameter value cannot be too large, it should be less than the image display time+image dark time - @param [in] CF_SRBY (Input) Input triggers effective edge,0 = rising edge;1 = falling edge - Example: If the user wants the Input triggers effective edge as the rising edge, this parameter = 0; - If the user wants the Input triggers effective edge as the falling edge,this parameter = 1; - @param [in] CF_SCBY (Input) Output triggers effective edge,0 = rising edge; 1 = falling edge - ；Example: If the user wants the output triggered effective edge as the rising edge, this parameter = 0; - If user wants the output triggered effective edge as the falling edge, this parameter = 1; - @param [in] CF_JG (Input) Output trigger interval, unit: binary graph, range: 1-65535 - Example: Assuming the user wants to output a trigger every 3 binary images, this parameter = 3; - Assuming the user wants to output a trigger for every 8-bit image, this parameter = 1 * 8; - @param [in] CF_YS (Input) Output trigger delay, unit: 5ns, range: 0-83886075ns - Example: Assuming the user wants to output a trigger delay of 10us, this parameter=10000/5=2000; - Tips:This parameter value should be less than the image display time + the image dark time |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

  - Function 12: Setting Image Display Parameters Function

- The function in the header file displays：

- FL_DLP_HS_F4321_SetParamPicShow(int DeviceID,int CMDID,int Time_Type,unsigned int Gray_Type,unsigned int Time_Show,unsigned int Time_Dark)

| - Function Function Description | - @brief Set image display parameters - exposure time parameter, dark time parameter, grayscale level parameter - Tips:The image display parameters set by this command are only applicable to normal cache playback mode |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] CMDID (Input) Command ID,Command ID = 21 - @param [in] Time_Type (Input) Type of time calculation，This parameter = 1 - @param [in] Gray_Type (Input) Image display grayscale level, range: 1-16 - Example: Assuming the user wants to play a binary image, this parameter = 1; Assuming the user wants to play an 8-bit image, this parameter = 8 - @param [in] Time_Show (Input) Image display exposure time, unit: 5ns - Example: Assuming the user wants to display a single image for 1 second, this parameter=1000000000/5=200000000 - Tips：When Gray_Type = 1, this parameter is the display time of a single binary graph; - When Gray_Typ e= 8, this parameter represents the display time of a single 8-bit image; - @param [in] Time_Dark (Input) Image display dark time, unit: 5ns - Example: Assuming the user wants to be dark for 1 second after a single image is displayed, this parameter=1000000000/5=200000000 |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

  - Function 13: Set DMD display parameter function

- The function in the header file displays：

- FL_DLP_HS_F4321_SetParamDMDShow(int DeviceID,int CMDID,unsigned int DMDShow_SP,unsigned int DMDShow_Height)

| - Function Function Description | - @brief Setting DMD display parameters - can achieve partial display function of DMD to improve playback frame rate |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] CMDID (Input) Command ID,Command ID = 22 - @param [in] DMDShow_SP (Input) DMD display starting line, range: 1- DMD resolution high - Example: Assuming the user wants to display from line 100 onwards, this parameter = 100 - @param [in] DMDShow_Height (Input) DMD display lines, range: 1- remaining display lines - Example: Assuming the user wants to display 500 rows, this parameter=500 - Tips：If the user wants to know the playback frequency of 50K and the maximum number of lines displayed, please contact the after-sales engineer - Tips:The default display of DMD is from top to bottom, starting from the DMDShow_SP and displaying the DMDShow_Height; If the DMD upside down display command is sent, it will start displaying from the DMDShow_SP and display the DMDShow_Height upwards. |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

  - Function 14: Set scrolling playback mode - Display parameter function

- The function in the header file displays：

- FL_DLP_HS_F4321_SetParamGDShow(int DeviceID,int CMDID,unsigned int GD_SP,unsigned int GD_DCBJH,unsigned int GD_BJC,int GD_Type)

| - Function Function Description | - @brief Set image scrolling mode display parameters |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] CMDID (Input) Command ID,Command ID = 23 - @param [in] GD_SP (Input) Scroll to play the starting line, - Tips:The storage of scrolling images in the device starts from the first line, meaning that every time the scrolling image data is sent, it will overwrite the original data. - Example: Assuming the user wants to start playing from line 100, this parameter=100 - @param [in] GD_DCBJH (Input) The number of scrolling steps per iteration, which means how many rows the DMD scrolls each time it refreshes the image. - Example: Assuming the user wants to scroll 10 lines at a time, this parameter=10 - @param [in] GD_BJC (Input) The total number of scrolling steps, which means the maximum number of scrolling times. - Example: Assuming the user wants to scroll up to 1000 times, this parameter is equal to 1000 - Tips:When DMD displays the first scrolling image, it will directly display an image equal to the DMD resolution high line count. Therefore, assuming that the number of scrolling steps per step is 1, the maximum total number of scrolling steps = (number of scrolling data lines - DMD resolution high - GD_SP + 1) - @param [in] GD_Type (Input) Scroll type 0 - Loop scrolling; 1 - Single scroll - Example: Assuming the user wants to scroll in a loop, this parameter = 0; - Assuming the user wants to scroll once, this parameter = 1; |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

  - Function 15: Set playback mode (HDMI real-time mode; Gigabit/10G real-time mode; Internal/External Trigger Rolling Mode) Function

- The function in the header file displays：

- FL_DLP_HS_F4321_Play2(int DeviceID,int CMDID,int PlayType)

| - Function Function Description | - Set the device to enter the corresponding playback mode - HDMI real-time mode; Gigabit/10G real-time mode; Scroll Mode |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] CMDID (Input) Command ID,Command ID = 24 - @param [in] PlayType (Input) Play type, range: 0, 1, 2, 3, 4 - 0 - HDMI real-time mode； - 1 - Gigabit Ethernet real-time mode； - 2 - 10 Gigabit Ethernet real-time mode； - 3 - Internally triggered scrolling playback mode； - 4 - External triggered scrolling playback mode； |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

  - Function 16: Rolling Play Mode - Internal Trigger Rolling Start Play Function

- The function in the header file displays：

- FL_DLP_HS_F4321_GDPlay(int DeviceID,int CMDID,unsigned int GD_Time,unsigned int GD_Num)

| - Function Function Description | - @brief Rolling playback mode - internal trigger rolling mode starts playback - Tips：This function is only applicable to internally triggered scrolling mode |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] CMDID (Input) Command ID,Command ID = 25 - @param [in] GD_Time (Input) Internal trigger scroll mode single image display time, unit: 5ns - Example: Assuming the user wants each scrolling image to display for 1 second, this parameter=1000000000/5=200000000 - @param [in] GD_Num (Input) Total number of trigger pulses for internal triggering of scrolling mode - Tips:This parameter needs to be less than or equal to the total number of rolling steps(GD_BJC) in function 14 |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |

  - 函数十七：发送图像 函数

- The function in the header file displays：

- FL_DLP_HS_F4321_SendPic(int DeviceID,int JX_Type,int Length_DZ,unsigned int JX_PicNum,int JX_PicFX,int JX_HC_SP,unsigned char * JX_BMPDir,unsigned char * JX_BinAddr,unsigned char *JX_CharDate)

| - Function Function Description | - @brief Send image function |
| --- | --- |
| - Function parameter description | - @param [in] DeviceID (Input) Device ID, Example: 1 - @param [in] JX_Type; (Input) 14 modes of parsing types - 0 - Binary BMP folder mode； - 1 - Binary Bin file mode； - 2 - Binary char array mode； - 3 - 8-bit BMP folder mode； - 4 - 8-bit Bin file mode； - 5 - 8-bit char array mode； - 6 - Binary BMP folder real-time mode； - 7 - Binary Bin file real-time mode； - 8 - Binary char array real-time mode； - 9 - 8-bit BMP folder real-time mode； - 10 - 8-bit Bin file real-time mode； - 11 - 8-bit char array real-time mode； - 12 - Rolling Image Bin File Mode； - 13 - Rolling image char array mode； - @param [in] Length_DZ; (Input) Single frame transmission data length, default value 1024, representing a single transmission of 1024 bytes for Ethernet data packets - @param [in] JX_PicNum; (Input) the number of images sent; - When the JX_Type is not Rolling Image, this parameter is the number of images sent. When JX_Type binary images, this parameter represents n binary images; When JX_Type 8-bit images, this parameter represents n 8-bit images; - When the JX_Type is Rolling Image, this parameter is the total number of rows in the scrolling image - @param [in] JX_PicFX; (Input) Image direction - 0 - Image forward display； 1 - Display the image upside down； - 2 - Mirror display of left and right images； - 3 - Display the image upside down, left and right reversed - Tips:The image direction parameter is only valid for related modes with BMP parsing type - @param [in] JX_HC_SP; (Input) Cache mode - Image starting storage location; Unit: Binary chart - The meaning of this parameter is to cache images starting from the nth binary image position of the lower computer device - Tips:This parameter is invalid when the parsing type is real-time or scrolling image;The cache of scrolling images starts from the first line data position of the device - @param [in] JX_BMPDir (Input) Resolve BMP folder address - When the parsing mode is BMP folder format, this parameter is used to input the absolute path of the BMP image folder; - When using other parsing modes, this parameter is left blank - Tips:The absolute path needs to be in full English and cannot have any Chinese - @param [in] JX_BinAddr; (Input) Resolve Bin file address - When the parsing mode is in Bin file format, this parameter is used to input the absolute path of the Bin image file - When using other parsing modes, this parameter is left blank - Tips:The absolute path needs to be in full English and cannot have any Chinese - @param [in] JX_CharDate; (Input) Parse char array format image data array - When the parsing mode is Char format, this parameter is used to input a pointer to a char type array - When using other parsing modes, this parameter is left blank - Tips：When using char array mode, the char array stores image data. The char array is a one-dimensional array, and storing image data in the char array follows the rule of image data from left to right and top to bottom. - When a user sends a binary image, a pixel only has two states, 0 or 1, and a byte is 8 bits, so one byte can store 8 pixels. The user needs to form a byte by bit for every 8 pixels, and the order of pixels is from left to right and top to bottom of the image. - Example: If 8 pixels are shown as 1 0 1 0 0 1 0, then the byte it represents is 0xB2 - When a user sends an 8-bit image, one pixel equals one byte, so simply assign the pixels to the char array from left to right and top to bottom of the image |
| - Function feedback | - @return ：When the return value of the function is 0, it is normal. Negative numbers have errors |