F4321_HS Dynamic Library

Usage Document

Time: 2025.2.25

- Simple explanation of the overall process of using dynamic libraries

- Step 1: Call the "Initialize UDP Network Communication" function.

- FL_DLP_HS_F4321_Init(int DeviceID,char* server_addr,int server_port,char* target_addr,int target_port)

- example：FL_DLP_HS_F4321_Init(1,“192.168.1.10”,1234,“192.168.1.100”,1234)

- Step 2: Call the "Send command without parameters" function to send the query command.

- FL_DLP_HS_F4321_CMDNoParam(int DeviceID,int CMDID)

- example：FL_DLP_HS_F4321_CMDNoParam(1,1)

- Step 3: Call the "Get Device Information" function.

- FL_DLP_HS_F4321_GetDeviceInfo(int DeviceID,unsigned char *DeviceInfo,int * DeviceInfolen)

- example：unsigned char *DeviceInfo = (unsigned char *)malloc(2048);

- int DeviceInfolen = 0;

- FL_DLP_HS_F4321_GetDeviceInfo(1,DeviceInfo,&DeviceInfolen)

- Print DeviceInfo as shown below

- Step 4: Determine whether initialization is successful based on device information

- Initialization successful: Users can freely call other functions according to their own needs

- Initialization failed: First try restarting the device. If it cannot be resolved, contact the after-sales engineer

- Step 5: Before powering off the equipment, call the function "Send command without parameters".

- FL_DLP_HS_F4321_CMDNoParam(int DeviceID,int CMDID)

- First，Send the playback termination command，example：FL_DLP_HS_F4321_CMDNoParam(1,3)

- Second，Send DMD Float command，example：FL_DLP_HS_F4321_CMDNoParam(1,4)

- Finally，the device loses power

- Step 6: Call the "Close UDP Network Communication" function

- FL_DLP_HS_F4321_DeInit(int DeviceID)

- example：FL_DLP_HS_F4321_DeInit(1)

- Tips:

- The first three steps must be executed each time the dynamic library is called, as the dynamic library needs to obtain the corresponding device calculation parameters from the device

- A delay of 1ms is required between every two instructions (the function for sending the image will be explained separately), as there is a delay from the PC sending the instruction to the device receiving the instruction to the end of the device executing the instruction

- The waiting time for the 'send image function' is determined by the amount of image data sent and the sending speed of the Ethernet port. The sending speed of the Ethernet port is related to the configuration of the PC, so the delay waiting time cannot be accurately calculated。It is recommended to call "Get the latest operation information - correct information function" after calling the "Send image function", print the operation information, and check whether the amount of received data in the feedback information of the device is the same as the amount of sent image data

- The DMD Float command is only called before the device is powered off, and it is not used during the operation of the device. If it is used, it must be powered off and restarted

- If the DMD is in playback mode and the user wants to reset parameters or send images, they must first send a "playback termination command" to put the DMD in playback termination mode, otherwise there will be errors in playing images, playback frequency, and other phenomena.

- If the user wants to use the trigger signal function of DMD, they need to call the "Set Trigger Parameter Function" every time the device is powered on again to reset the trigger related parameters

- Normal Cache Mode Playback - Example Process

- Normal Cache Mode Description:

- The order of images in BMP folder/the order of image data in Bin/the order of image data in Char array, which refers to the order in which images are stored in the device after being sent, that is, the order of images when playing DMD. This mode is to play and display images in this order.

- Tips:The BMP image names in the BMP folder need to be of equal length and ordered, for example: 000000.bmp; 000001.bmp;......; 000011.bmp;Otherwise, there may be a phenomenon of incorrect image order

- The first four steps and the last two steps are the same as (一． Simple explanation of the overall process of using dynamic libraries), and will not be described again

- Step 4: Call the "Set Trigger Parameters" function.

- If the user wants to use the triggering function of this device, this parameter needs to be set, otherwise it can be skipped

- FL_DLP_HS_F4321_SetParamCF(int DeviceID,int CMDID,unsigned int CF_KD,int CF_SRBY,int CF_SCBY,int CF_JG,int CF_YS)

- example：

- CF_KD：Unit: 5ns，This parameter is used to set the duration of each output trigger signal

- 10us,This parameter=10000/5=2000

- CF_SRBY：0- Rising edge trigger; 1- Falling edge trigger

- CF_SCBY：0- Rising edge trigger; 1- Falling edge trigger

- CF_JG：1，Output a trigger signal for each binary image played

- CF_YS：0

- FL_DLP_HS_F4321_SetParamCF(1,20,2000,0,0,1,0)

- Step 5: Call the "Send Image" function. Users can choose to send BMP image folders, Bin files, or Char arrays according to their own needs

- FL_DLP_HS_F4321_SendPic(int DeviceID,int JX_Type,int Length_DZ,unsigned int JX_PicNum,int JX_PicFX,int JX_HC_SP,unsigned char * JX_BMPDir,unsigned char * JX_BinAddr,unsigned char *JX_CharDate)

- JX_Type：

- 0 - binary BMP folder;

- 1 - Binary Bin file;

- 2 - Binary char array;

- 3 - 8 bit BMP folder;

- 4 - 8 bit Bin file;

- 5 - 8 bit char array;

- Example of binary BMP image folder:

- Length_DZ: 1024 byte

- JX_PicNum: 10；

- JX_PicFX:

- 0 - Image positive image;

- 1 - Invert the image vertically;

- 2 - Invert the image left and right;

- 3 - Invert the image up, down, left, and right

- JX_HC_SP: 1,This parameter represents storing images starting from the position of the first binary image in the device

- JX_BMPDir: I:\testpic；

- FL_DLP_HS_F4321_SendPic(1,0,1024,10,0,1,“I:\testpic”,NULL,NULL)

- Example of binary Bin file:

- Length_DZ: 1024 byte

- JX_PicNum: 10；

- JX_PicFX: This parameter is invalid for Bin format, default value is 0;

- JX_HC_SP: 1；

- JX_BinAddr: I:\testpic\test.bin；

- FL_DLP_HS_F4321_SendPic(1,1,1024,10,0,1,NULL,“I:\testpic\test.bin”,NULL)

- Example of binary Char array:

- Length_DZ: 1024 byte

- JX_PicNum: 10；

- JX_PicFX: This parameter is invalid for Bin format, default value is 0;

- JX_HC_SP: 1；

- FL_DLP_HS_F4321_SendPic(1,2,1024,10,0,1,NULL,NULL,JX_CharDate)

- Step 6: Call the "Set Image Display Parameters" function.

- FL_DLP_HS_F4321_SetParamPicShow(int DeviceID,int CMDID,int Time_Type,unsigned int Gray_Type,unsigned int Time_Show,unsigned int Time_Dark)

- example：

- Time_Type：The default value for this parameter is 1

- Gray_Type：1，DMD plays images in binary image mode

- Time_Show：1s,This parameter=1000000000/5=200000000

- Time_Dark：1s,This parameter=1000000000/5=200000000

- FL_DLP_HS_F4321_SetParamPicShow(1,21,1,1,200000000,200000000)

- Step 7: Call the "Normal Cache Mode/Variable Sequence Mode Start Play" function

- FL_DLP_HS_F4321_Play1(int DeviceID,int PlayType,int Play_SP,int Play_PicNum)

- example：

- PlayType：

- 10- Internal single play; 11- Internal loop playback;

- 12- External single play;13- External loop playback；

- Play_SP：1，Starting from the first binary image

- Play_PicNum：Play 10 binary images

- FL_DLP_HS_F4321_Play1(1,10,1,10)

- Tips：When the DMD is in playback mode, if the user wants to resend image data or reset playback parameters, they must first send a playback termination command to put the DMD in playback termination mode, and then resend image data or reset playback parameters. Otherwise, there may be image errors, playback frequency errors, and other phenomena.

- Variable sequence mode playback - example process

- Variable sequence playback mode description:

- The user first stores the image data in normal cache mode, and then sets a variable sequence according to their own needs, similar to: the second image displays for 1 second, dark for 0 seconds; The eighth image shows 2 seconds, darkness for 1 second;......You can freely choose to play some of the cached images. The playback order, display time, and post display dark time of images are determined by a variable sequence set by the user, such as the first one playing the second image and the second one playing the eighth image. And the display time and dark time of each image can also be flexibly set

- Tips:Variable sequence mode can only play binary images

- The first five steps and the last two steps are the same as (二.Normal Cache Mode Playback - Example Process), and will not be described again

- Step 6: Call the set device variable sequence function

- FL_DLP_HS_F4321_SetKBXL(int DeviceID,int CMDID,int Sequence_num,int Time_type,unsigned int * Sequence_Add,unsigned int * Sequence_ShowTime,unsigned long long * Sequence_DarkTime)

- example：

- Sequence_num: 3,This means that there are only 3 binary images in the variable sequence in total

- Time_type: 1,This parameter can be set to the default value of 1

- Sequence_Add: Unit: Binary image, starting from 1

- Sequence 1 - Second image; Sequence 2 - Fourth image; Sequence 3 - Sixth image;

- Sequence_Add[0] = 2；Sequence_Add[1] = 4；Sequence_Add[2] = 6；

- Sequence_ShowTime: Unit: 5ns,Display time of sequence images

- Sequence 1 - display for 1s; Sequence 2 - display for 2s; Sequence 3 - Display for 3s

- Sequence_ShowTime[0] = 200000000；

- Sequence_ShowTime[1] = 400000000；

- Sequence_ShowTime[2] = 600000000；

- Sequence_DarkTime: Unit: 5ns,Dark time after image display

- Sequence 1 - Dark time 1s; Sequence 2 - Dark time 2s; Sequence 3- Dark time 3s;

- Sequence_DarkTime[0] = 200000000；

- Sequence_DarkTime[1] = 400000000；

- Sequence_DarkTime[2] = 600000000；

- FL_DLP_HS_F4321_SetKBXL(1,9,3,1,Sequence_Add,Sequence_ShowTime, Sequence_DarkTime)

- Step 7: Call the "Normal Cache Mode/Variable Sequence Mode Start Play" function

- FL_DLP_HS_F4321_Play1(int DeviceID,int PlayType,int Play_SP,int Play_PicNum)

- example：

- PlayType:

- 14- Variable sequence internal single play;

- 15- Variable sequence internal loop playback;

- 16- Variable sequence external single play;

- 17- Variable sequence external loop playback;

- Play_SP: This parameter is not valid for variable sequences,set to the default value of 1

- Play_PicNum: This parameter is not valid for variable sequences,set to the default value of 1

- FL_DLP_HS_F4321_Play1(1,14,1,1)

- Tips:The number of playback images, playback order, and playback time parameters of the variable sequence are completely determined by the "Set Device Variable Sequence Function"

- Rolling playback mode - Example process

- Rolling playback mode description:

- Rolling image data: a binary image of DMD resolution width multiplied by the number of rows in the rolling image. The image sending function temporarily supports sending data in Bin format or Char array format. This mode allows the image to achieve the effect of scrolling up and down.

- The first four steps and the last two steps are the same as (一． Simple explanation of the overall process of using dynamic libraries), and will not be described again

- Step 5: Call the "Send Image" function. Users can choose to scroll Bin files or scroll Char arrays according to their own needs

- FL_DLP_HS_F4321_SendPic(int DeviceID,int JX_Type,int Length_DZ,unsigned int JX_PicNum,int JX_PicFX,int JX_HC_SP,unsigned char * JX_BMPDir,unsigned char * JX_BinAddr,unsigned char *JX_CharDate)

- JX_Type: 12- Rolling Image Bin File；13- Rolling image char array；

- Example of Rolling Image Bin File:

- Length_DZ: 1024byte

- JX_PicNum: 1500, the scrolling image has 1500 rows of data;

- Tips:When the parsing type is scrolling, this parameter is the total number of rows in the scrolling image

- JX_PicFX: This parameter is invalid for Bin format, default value is 0;

- JX_HC_SP: Set as default value 1；

- Tips:JX_HC_SP This parameter is invalid when the parsing type is real-time or scrolling;

- The cache of scrolling images starts from the first line data position of the lower level device

- JX_BinAddr: I:\testpic\GD_test.bin；

- FL_DLP_HS_F4321_SendPic(1,12,1024,1500,0,1,NULL,“I:\testpic\GD_test.bin”,NULL)

- Example of scrolling image Char array:

- Length_DZ: 1024byte

- JX_PicNum: 1500, the scrolling image has 1500 rows of data;

- JX_PicFX: This parameter is invalid for Char Array format, default value is 0;

- JX_HC_SP: Set as default value 1；

- FL_DLP_HS_F4321_SendPic(1,13,1024,1500,0,1,NULL,NULL,JX_CharDate_GD)

- Step 6: Call the "Set Rolling Play Mode - Display Parameters" function

- FL_DLP_HS_F4321_SetParamGDShow(int DeviceID,int CMDID,unsigned int GD_SP,unsigned int GD_DCBJH,unsigned int GD_BJC,int GD_Type)

- example：

- GD_SP: 1，Play from the first line of the scrolling image

- GD_DCBJH: 1，Scroll one line at a time

- GD_BJC: 1000，Scroll 1000 times in total

- GD_Type: 0- Loop scrolling; 1- Single scroll

- FL_DLP_HS_F4321_SetParamGDShow(1,23,1,1,1000,1)

- Step 7: Call "Set playback mode (HDMI real-time mode; Gigabit/10G real-time mode; Internal/External Trigger Rolling Mode "Function

- FL_DLP_HS_F4321_Play2(int DeviceID,int CMDID,int PlayType)

- example：

- PlayType:  3 - Internally triggered scrolling playback mode；

- 4 - External triggered scrolling playback mode；

- FL_DLP_HS_F4321_Play2(1,24,3)

- Step 8: Call the "Rolling Play Mode - Internal Trigger Rolling Start Play" function

- Tips:If the user wants to use the internal trigger scrolling mode, they need to perform this step; If using external trigger scrolling mode, this step is not required

- FL_DLP_HS_F4321_GDPlay(int DeviceID,int CMDID,unsigned int GD_Time,unsigned int GD_Num)

- example：

- GD_Time: 1s,Each scrolling image displays for 1 second, This parameter = 200000000

- GD_Num: 500，Internal trigger scrolling 500 times

- Tips:GD_Num needs to be less than or equal to the total number of rolling steps in function 14

- FL_DLP_HS_F4321_GDPlay(1,25,200000000,500)

- HDMI Real time Mode Playback - Example Process

- HDMI Real time Mode Description:

- This mode requires connecting the HDMI port of the PC and the device, and directly outputting the display image of the PC monitor through the HDMI port. HDMI real-time mode displays 8-bit grayscale images.

- The first four steps and the last two steps are the same as (一． Simple explanation of the overall process of using dynamic libraries), and will not be described again

- Step 5: Call the "Set HDMI Real time Playback Mode Parameters" function

- FL_DLP_HS_F4321_SetParamHDMI(int DeviceID,int CMDID,unsigned int Time_Show)

- example：

- Time_Show: 10ms,This parameter=10000000/5=2000000

- FL_DLP_HS_F4321_SetParamHDMI(1,18,2000000)

- Step 6: Call "Set playback mode (HDMI real-time mode; Gigabit/10G real-time mode; Internal/External Trigger Rolling Mode "Function

- FL_DLP_HS_F4321_Play2(int DeviceID,int CMDID,int PlayType)

- example：

- PlayType: 0 - HDMI real-time mode；

- FL_DLP_HS_F4321_Play2(1,24,0)

- Gigabit/10 Gigabit Ethernet real-time mode playback - example process

- Gigabit/10 Gigabit Ethernet Real time Mode Description:

- This mode requires connecting the network card interface between the PC and the device, and the user can decide whether to use a gigabit network port or a ten gigabit network port. This mode will directly output the image to the DMD for display, and the process of sending the image is the process of displaying the image. Therefore, the speed of image switching depends on the speed of network transmission.

- The first four steps and the last two steps are the same as (一． Simple explanation of the overall process of using dynamic libraries), and will not be described again

- Step 5: Call the "Set Gigabit/10G Ethernet real-time playback parameters" function

- FL_DLP_HS_F4321_SetParamSS(int DeviceID,int CMDID,int GrayMode,int Time_type,unsigned int Time_Show)

- example：

- GrayMode: 0-binary image; 1-8 bit grayscale image

- Time_type:This parameter is temporarily useless and set to the default value of 1

- Time_Show:This parameter is temporarily useless and set to the default value of 0

- FL_DLP_HS_F4321_SetParamSS(1,19,0,1,0)

- Step 6: Call "Set playback mode (HDMI real-time mode; Gigabit/10G real-time mode; Internal/External Trigger Rolling Mode "Function

- FL_DLP_HS_F4321_Play2(int DeviceID,int CMDID,int PlayType)

- example：

- PlayType: 1-Gigabit Ethernet real-time mode; 2-10 Gigabit Ethernet real-time mode;

- FL_DLP_HS_F4321_Play2(1,24,1)

- Step 7: Call the "Send Image" function

- FL_DLP_HS_F4321_SendPic(int DeviceID,int JX_Type,int Length_DZ,unsigned int JX_PicNum,int JX_PicFX,int JX_HC_SP,unsigned char * JX_BMPDir,unsigned char * JX_BinAddr,unsigned char *JX_CharDate)

- JX_Type:

- 6- Real time binary BMP folder; 7- Real time binary Bin file; 8- Real time binary char array; 9- Real time 8-bit BMP folder; 10- Real time 8-bit Bin file; 11- Real time 8-bit char array;

- Example of binary BMP image folder:

- Length_DZ: 1024 bytes

- JX_PicNum: 10;

- JX_PicFX: Forward -0;

- JX_HC_SP: Real time mode, this parameter is invalid, set to default value 1;

- JX_BMPDir: I:\testpic；

- FL_DLP_HS_F4321_SendPic(1,6,1024,10,0,1,“I:\testpic”,NULL,NULL)

- Example of binary Bin file:

- Length_DZ: 1024 bytes

- JX_PicNum: 10;

- JX_PicFX: This parameter is invalid for Bin, default value is 0;

- JX_HC_SP: Real time mode, this parameter is invalid, set to default value 1;

- JX_BinAddr: I:\testpic\test.bin；

- FL_DLP_HS_F4321_SendPic(1,7,1024,10,0,1,NULL,“I:\testpic\test.bin”,NULL)

- Example of binary Char array:

- Length_DZ: 1024 bytes

- JX_PicNum: 10;

- JX_PicFX: This parameter is invalid for Char arrays, default value is 0;

- JX_HC_SP: Real time mode, this parameter is invalid, set to default value 1;

- FL_DLP_HS_F4321_SendPic(1,8,1024,10,0,1,NULL,NULL,JX_CharDate)