F4321_HS Dynamic Library_Labview

Time: 2025.2.25

- Labview imports dynamic libraries

- Step 1: Use Labview 2020 64 bit English version, click to import dynamic library

- Step 2: Choose to create a new dynamic library VI

- Step 3: Select the dynamic library DLL file and the dynamic library h header file

- Tips：Chinese cannot exist in the header file, otherwise the software will crash and crash

- Step 4: On this interface, add some definitions:

- F4321_HS_LIBRARY;WIN32;WIN64;_MSC_VER;_WIN32;_WIN64;__NT__;__WIN32__;__WIN64__;__cplusplus;

- Step 5: On this interface, confirm whether the number of dynamic library functions and function names are correct

- Step 6: Define the name and storage location of the dynamic library VI

- Step 7: Click next

- Step 8: Adjust the parameters of some functions

- FL_DLP_HS_F4321_SendPic（）：JX_CharDate

- Array size can be adjusted by users themselves，

- Array Size = DMD resolution width * DMD resolution height/8 * bit depth * number of images

- FL_DLP_HS_F4321_SetKBXL（）：

- Sequence_Add，Sequence_ShowTime，Sequence_DarkTime

- Step 9: Click next

- Step 10: Click next

- Step 11: Click Finish

- The generated dynamic library VI is located in the storage path of step six.

- Labview directly uses the dynamic library VI we provide

- Put the generated dynamic library VI folder directly into the Vi folder of Labview software, and then open Labview software to directly call the dynamic library VI function

- Dynamic library VI folder:

- Labview software VI folder path: